倒立摆实验报告

课程名称:	                  强化学习

报告题目:	               倒立摆实验报告

学生姓名:	                   李昊宸

报告时间:	              2022年4月11日

目  录

第1章 模型描述	1

1.1 问题介绍	1

1.2 数学模型	1

第2章 研究思路	3

2.1 离散化奖励模型	3

2.2 离散化状态空间	4

2.3 离散化动作空间	4

第3章 方法实现	5

3.1 Q-Learning	5

3.1.1 目标更新	5

3.1.2 算法实现	5

3.2 SARSA	7

3.2.1 目标更新	7

3.2.2 算法实现	8

第4章 实验结果	10

4.1 Q-Learning	10

4.2 SARSA	12

4.3 Q-Learning与SARSA的学习过程对比	14

模型描述

问题介绍

倒立摆是将一个物体固定在一个圆盘的非中心点位置，由直流电机驱动将其在垂直平面内进行旋转控制的系统(图1)。由于输入电压是受限的,电机并不能提供足够的动力直接将摆杆推完一圈。相反,需要来回摆动收集足够的能量，然后才能将摆杆推起并稳定在最高点。

图1 倒立摆问题

数学模型

表1给出了倒立摆的物理系统参数：

表1 倒立摆问题系统参数

根据参数，可以建立倒立摆系统的连续时间动力学模型：

其中系统的状态为二维组合。角，角速度，电压。

def get_diff_diff_alpha(alpha, diff_alpha, u):

m = 0.055

g = 9.81

l = 0.042

J = 1.91e-4

b = 3e-6

K = 0.0536

R = 9.5

return (m*g*l*np.sin(alpha) - b*diff_alpha-(K**2)*diff_alpha/R + K*u/R)/J

研究思路

  - 离散化奖励模型

为将连续时间离散化，我们设置倒立摆的采样时间。在此基础上，倒立摆系统的离散时间动力学模型由下式给出：

def get_new_state(old_alpha, old_diff_alpha, action):

Ts = 0.005

new_alpha = old_alpha + Ts * old_diff_alpha

new_diff_alpha = old_diff_alpha + Ts * get_diff_diff_alpha(old_alpha, old_diff_alpha, action)

#速度限制：[-15pi，15pi]

max_diff_alpha = 15 * np.pi

if new_diff_alpha < -max_diff_alpha:

new_diff_alpha = -max_diff_alpha

elif new_diff_alpha > max_diff_alpha:

new_diff_alpha = max_diff_alpha

return new_alpha, new_diff_alpha

控制目标是将摆杆从最低点摆起并稳定在最高点。奖励函数定义成如下二次型形式：

展开后也即：

最大化该奖励函数的过程，也就是最小化摆点到平衡位置的角度差，同时在步进过程中，将角加速度和电压约束到尽量小，以实现系统的稳定。

def get_reward(alpha, diff_alpha, action):

Rrew = 1

return -(5*alpha**2+0.1*diff_alpha**2)-Rrew*action**2

离散化状态空间

采用离散化法，划分连续的角度空间和角速度空间。

将角度空间划分为连续且独立的200份：

将角速度空间也划分为连续且独立的200份：

据此，构建离散状态空间

def get_indice(alpha, diff_alpha, num_alpha, num_diff_alpha):

max_diff_alpha = 15 * np.pi

#正则化alpha，范围[-pi，pi)

norm_alpha = (alpha + np.pi) % (2 * np.pi) - np.pi

#alpha下标范围[0,num_alpha-1]

indice_alpha = int((norm_alpha + np.pi)/(2*np.pi) * num_alpha)

#diff_alpha下标范围[0,num_diff_alpha-1]

indice_diff_alpha = int((diff_alpha + max_diff_alpha)/(2*max_diff_alpha) * num_diff_alpha)

if indice_diff_alpha == num_diff_alpha:

indice_diff_alpha -= 1

return indice_alpha, indice_diff_alpha

  - 离散化动作空间

采用离散的有限动作集，选取3个代表性动作。

actions = np.array([-3, 0, 3])

方法实现

  - Q-Learning

首先，采用Q-Learning方法求解该模型。

目标更新

Q-Learning是一种离策略学习：选取行动采用ε-greedy(Q)策略，而更新目标的策略使用greedy(Q)。

迭代中，在每一步长的探索中，智能体通过ε-greedy(Q)策略选取当前状态对应的动作，并据此生成数据。

更新Q函数时，以贪心策略的Q值作为时间差分目标，即：

据此，得到Q函数的迭代公式：

算法实现

def train(num_alpha, num_diff_alpha):

#状态限制

min_alpha, max_alpha = -np.pi, np.pi

min_diff_alpha, max_diff_alpha = -15 * np.pi, 15*np.pi

#Q动作状态-价值函数

Q = np.zeros([3, num_alpha, num_diff_alpha])

#离散动作

actions = np.array([-3, 0, 3])

# 迭代次数

N = 20000

#ε-greedy超参

epsilon = 1.0

epsilon_limit = 0.01

# 初始学习率

lr = 1.0

#衰减率

decay = 0.9995

#折扣因子

gamma = 0.98

# 初始状态

state_alpha, state_diff_alpha = -np.pi, 0

total_step = 300

optimal = -1e7

finalerror = 2*np.pi

total_angle = np.pi

# 探索步长限制

step_control = 300

#收敛目标控制

stable_target = 2

converted_alpha, converted_diff_alpha = 0.05, 0.01

#迭代

for i in range(N):

#每一轮迭代初始化

alpha, diff_alpha, iteration, total_reward = state_alpha, state_diff_alpha, 0, 0

#真实坐标到离散状态空间转换

indice_alpha, indice_diff_alpha = env.get_indice(alpha, diff_alpha, num_alpha, num_diff_alpha)

#收敛控制变量

stable_state = 0

#更新ε和学习率

epsilon = max(decay * epsilon, epsilon_limit)

lr = decay * lr

angles, total_acts = [], []

angles.append(alpha)

error_a = np.abs((alpha-min_alpha) % (max_alpha - min_alpha) + min_alpha)

#开始迭代

while iteration < step_control:

iteration += 1

#Π=ε-greedy(Q)采样动作at ~ Π(st)

greedy_action = env.get_greedy_action(Q, indice_alpha, indice_diff_alpha, epsilon)

total_acts.append(actions[greedy_action])

#执行动作，获取观测量rt+1，st+1

new_alpha, new_diff_alpha = env.get_new_state(alpha, diff_alpha, actions[greedy_action])

error = np.abs((new_alpha-min_alpha) % (max_alpha - min_alpha) + min_alpha)

if error < error_a:

min_angle = new_alpha

error_a = error

#收敛控制

if error_a < converted_alpha and np.abs(diff_alpha) < converted_diff_alpha:

angles.append(new_alpha)

stable_state += 1

#如果误差小于收敛阈值，并且连续保持收敛状态，即稳定在最高点

if stable_state == stable_target:

break

#新状态到状态空间转换

indice_new_alpha, indice_new_diff_alpha = env.get_indice(new_alpha, new_diff_alpha, num_alpha, num_diff_alpha)

#greedy(Q)贪心策略选取用于更新的动作

max_newQa = np.max(Q[:, indice_new_alpha, indice_new_diff_alpha])

#获取更新动作的奖励值

reward = env.get_reward(alpha, diff_alpha, actions[greedy_action])

#更新Q

deta = reward + gamma * max_newQa - Q[greedy_action][indice_alpha][indice_diff_alpha]

Q[greedy_action][indice_alpha][indice_diff_alpha] += lr * deta

alpha, diff_alpha, indice_alpha, indice_diff_alpha = new_alpha, new_diff_alpha, indice_new_alpha, indice_new_diff_alpha

angles.append(alpha)

total_reward += reward

total_step = iteration if iteration < total_step else total_step

min_angle = np.abs((min_angle + np.pi) % (2 * np.pi) - np.pi)

total_angle = min_angle if min_angle < total_angle else total_angle

SARSA

Q-Learning是离策略学习，在更新时永远选择奖励最高的动作，不考虑带来的其他后果，因此比较激进。为作比较，我们再采用SARSA方法求解该模型。

    - 目标更新

SARSA是一种在策略学习：选取行动和更新目标的策略都使用ε-greedy(Q)。

迭代中，在每一步长的探索中，智能体执行当前状态对应的动作，据此生成数据。

随后，根据策略ε-greedy(Q)采样动作，并根据更新Q，即：

    - 算法实现

def train(num_alpha, num_diff_alpha):

#状态限制

min_alpha, max_alpha = -np.pi, np.pi

min_diff_alpha, max_diff_alpha = -15 * np.pi, 15*np.pi

#Q动作状态-价值函数

Q = np.zeros([3, num_alpha, num_diff_alpha])

#离散动作

actions = np.array([-3, 0, 3])

# 迭代次数

N = 20000

#ε-greedy超参

epsilon = 1.0

epsilon_limit = 0.01

# 初始学习率

lr = 1.0

#衰减率

decay = 0.9995

#折扣因子

gamma = 0.98

# 初始状态

state_alpha, state_diff_alpha = -np.pi, 0

total_step = 300

optimal = -1e7

finalerror = 2*np.pi

total_angle = np.pi

# 探索步长限制

step_control = 300

#收敛目标控制

stable_target = 2

converted_alpha, converted_diff_alpha = 0.05, 0.01

#迭代

for i in range(N):

#每一轮迭代初始化

alpha, diff_alpha, iteration, total_reward = state_alpha, state_diff_alpha, 0, 0

#真实状态到离散状态空间转换

indice_alpha, indice_diff_alpha = env.get_indice(alpha, diff_alpha, num_alpha, num_diff_alpha)

#Π=ε-greedy(Q)，根据该策略采样动作at~Π(st)

greedy_action = env.get_greedy_action(Q, indice_alpha, indice_diff_alpha, epsilon)

#ε和学习率更新

epsilon = max(decay * epsilon, epsilon_limit)

lr = decay * lr

angles, total_acts = [], []

angles.append(alpha)

error_a = np.abs((alpha-min_alpha) % (max_alpha - min_alpha) + min_alpha)

while iteration < step_control:

iteration += 1

total_acts.append(actions[greedy_action])

#根据st，at获取新状态st+1

new_alpha, new_diff_alpha = env.get_new_state(alpha, diff_alpha, actions[greedy_action])

error = np.abs((new_alpha-min_alpha) % (max_alpha - min_alpha) + min_alpha)

if error < error_a:

min_angle = new_alpha

error_a = error

#收敛控制

if error_a < converted_alpha and np.abs(diff_alpha) < converted_diff_alpha:

angles.append(new_alpha)

stable_state += 1

print(stable_state)

if stable_state == stable_target:

break

#新状态映射到离散状态空间

indice_new_alpha, indice_new_diff_alpha = env.get_indice(new_alpha, new_diff_alpha, num_alpha, num_diff_alpha)

#在策略更新：根据st+1选择下一步动作at+1

greedy_new_action = env.get_greedy_action(Q, indice_new_alpha, indice_new_diff_alpha, epsilon)

#计算执行at到达st+1的奖励

reward = env.get_reward(alpha, diff_alpha, actions[greedy_action])

#根据st+1和at+1和reward更新Q(st,at)

update_Q = reward + gamma * Q[greedy_new_action][indice_new_alpha][indice_new_diff_alpha] - Q[greedy_action][indice_alpha][indice_diff_alpha]

Q[greedy_action][indice_alpha][indice_diff_alpha] += lr * update_Q

#步进

alpha, diff_alpha, indice_alpha, indice_diff_alpha, greedy_action = new_alpha, new_diff_alpha, indice_new_alpha, indice_new_diff_alpha, greedy_new_action

angles.append(alpha)

total_reward += reward

total_step = iteration if iteration < total_step else total_step

min_angle = np.abs((min_angle + np.pi) % (2 * np.pi) - np.pi)

total_angle = min_angle if min_angle < total_angle else total_angle

实验结果

Q-Learning

Q-Learning方法在3900轮迭代时，出现第一次收敛，收敛步长为262。经过20000轮迭代后，最小收敛步长为167，收敛时最终距离稳定状态角度的误差为0.018567 rad。

下面对Q函数进行了可视化，为了视觉效果，对Q函数进行了取反。

图2 Q-Learning Q动作-状态函数可视化（动作取-3V）

图3 Q-Learning Q动作-状态函数可视化（动作取0V）

图4 Q-Learning Q动作-状态函数可视化（动作取3V）

可以看出，无论选取哪个动作，在初始状态附近（如）时，Q值达到峰值，这与初始状态的奖励最低常识相吻合。并且，Q函数以目标状态（）为中心，形成了鞍点。

下面对于状态空间，在Q函数上采取greedy策略，将动作-状态选择进行了可视化：

图5 Q-Learning greedy策略下的Q动作-状态函数可视化

可以看出，动作与存在明显的分界线。理论上，动作应分布在分界线附近，但是由于Q-Learning较为激进的学习策略，学习到的结果倾向于最快速度的到达平衡位置，尽量不采取对状态影响最小的动作。

SARSA

SARSA方法在6700轮迭代时，出现第一次收敛，收敛步长为297。经过20000轮迭代后，最小收敛步长为292，收敛时最终距离稳定状态角度的误差为0.016288 rad。

图6 SARSA Q动作-状态函数可视化（动作取-3V）

图7 SARSA Q动作-状态函数可视化（动作取0V）

图8 SARSA Q动作-状态函数可视化（动作取3V）

可以看出，无论选取哪个动作，在初始状态附近（如）时，Q值达到峰值，这与初始状态的奖励最低常识相吻合。相比于Q-Learning的Q函数，SARSA的Q函数更为陡峭，这与SARSA的迭代步长相对更长相吻合。

下面对于状态空间，在Q函数上采取greedy策略，将动作-状态选择进行了可视化：

图9 Q-Learning greedy策略下的Q动作-状态函数可视化

可以看出，动作与存在明显的分界线。并且，动作的出现数量明显高于Q-Learning的决策。这也证实了SARSA更趋向于“安全”“稳定”的动作选择。

  - Q-Learning与SARSA的学习过程对比

图10给出了前7000次迭代中，Q-Learning与SARSA方法在停止本轮迭代时的最小loss。

图10 Q-Learning与SARSA前7000轮迭代的loss变化

可以看出，Q-Learning方法的loss下降更快，收敛也更快，这与Q-Learning每次都选取当前的贪心策略，不在乎之后是否会带来危险，以及SARSA每次都要多尝试一步的ε-贪心策略，更加保守的做出决策相吻合。

另外，我们对最终Q-Learning和SARSA的决策控制进行了可视化。视频可访问以下链接：

Q-Learning：https://github.com/Therock90421/21-22Reinforcement_learning_experiements/blob/main/inverted_pendulum/Q_Learning.gif

SARSA：https://github.com/Therock90421/21-22Reinforcement_learning_experiements/blob/main/inverted_pendulum/SARSA.gif